JACE Startup Checklist

New Install

- Create new system passphrase, username, and password

  - Default credentials admin/Niagara

- Enter network info if available Platform ->TCP/IP Configuration

- Enable wifi if applicable

  - Set mode to access point

  - Set password

  - Set timeout to 0

- Commission JACE -> Platform -> Platform Administration

  - Load license file

  - Select station to load

  - Set system time

  - Allow install to be completed/reboot

- Install distribution file -> Platform ->Distribution File Installer

  - Ensure directory at top of window is C:/Niagra/VerisionNumber/Distech Control Files. If not, navigate to that directory using the “Choose Directory” button at bottom.

  - Select appropriate file for your JACE

  - Allow system to reboot. This will take longer than normal.

Updating JACE

- If applicable, purchase a new license

- Back Up JACE -> Platform -> Station Copier

- Wipe JACE-> Platform ->Distribution File Installer

  - Ensure directory at top of window si C:/Niagra/VerisionNumber/CleanDist. If not, navigate to that directory using the “Choose Directory” button at bottom.

  - Select appropriate file for your JACE

  - Allow system to reboot. This will take longer than normal.

- From this point, the process continues like a fresh install.